ENGINEERING TEST REPORT

# PCB-001 KiCad Simulation Results

Adjustable NE555 astable LED flasher | transient characterization

Project: PCB-001 adjustable NE555 astable LED flasher

Record date: 2026-08-21

Tool: KiCad Simulator; application version not recorded

Evidence state: Simulation characterization complete; physical validation pending

# Scope and status

This report records transient-simulation characterization of the PCB-001 timing range and loaded LED branch. The KiCad simulation schematic is separate from the production schematic.

STATUS  Evidence is recorded, but no requirement is marked PASS. Formal acceptance criteria were not documented before the run, and simulation does not close fabrication, assembly, or physical-validation gates.

# Simulation configuration

| Parameter | Configuration |
| --- | --- |
| Analysis | Transient; PSpice/LTspice compatibility enabled |
| Supply | 5 V DC |
| Timer model | External uA555.sub, subcircuit UA555; symbol pins 1-8 mapped to model pins 1-8 |
| Timing network | R1 = 10 kOhm, R2 = 10 kOhm, C1 = 10 uF |
| Adjustment | R4 substitutes for RV1; evaluated at 1 Ohm, 50 kOhm, and 100 kOhm |
| Timing runs | D1/R3 excluded; 100 us step and maximum step; 2 s or 10 s duration |
| Loaded run | R4 = 50 kOhm; D1 and R3 = 330 Ohm included; 2 s duration |
| LED model | Generic built-in diode: is = 1e-18 A, rs = 5 Ohm, n = 2; not manufacturer-specific |

# Timing-range characterization

R4 was swept across the simulated adjustment range. The 1 Ohm case approximates the minimum setting while avoiding a zero-ohm SPICE element.

| R4 | Period | Frequency | High / low | Duty | VCAP range |
| --- | --- | --- | --- | --- | --- |
| 1 Ohm | 0.197 s | 5.08 Hz | 0.127 / 0.070 s | 64.5% | approx. 1.85-3.38 V |
| 50 kOhm | 0.845 s | 1.18 Hz | 0.443 / 0.402 s | 52.4% | approx. 1.85-3.38 V |
| 100 kOhm | 1.45 s | 0.690 Hz | 0.767 / 0.683 s | 52.9% | approx. 1.85-3.38 V |

Observation: saved CSV data shows VCAP repeatedly traversing approximately one-third to two-thirds of the 5 V supply while VOUT toggles.

Figure 1. Midrange timing response at R4 = 50 kOhm: VOUT toggles as VCAP ramps between the timer thresholds.

Brief switching spikes seen in some VOUT exports are treated as external UA555-model or numerical switching artifacts, not as expected steady hardware output levels.

# Loaded LED-branch characterization

The LED branch was restored at the 50 kOhm timing setting. D1 used the documented generic red-LED diode parameters and R3 remained 330 Ohm.

| Quantity | Recorded result |
| --- | --- |
| Steady VOUT high level | approximately 3.62 V |
| D1 cathode / R3 input node | approximately 1.72 V while on |
| LED forward voltage | approximately 1.90 V |
| LED on-current | approximately 5.20 mA |
| R3 steady on-state power | approximately 8.92 mW |
| R3 maximum saved transient power | 10.19 mW |

Consistency check. 1.72 V / 330 Ohm = 5.21 mA, agreeing with the simulated 5.20 mA LED current.

Figure 2. Loaded LED voltages at R4 = 50 kOhm: VOUT and the D1 cathode / R3 input node.

# LED current and resistor dissipation

The LED current settles near 5.20 mA during each on interval. The narrow transition excursion is retained in the evidence but is not used as the steady operating current.

Figure 3. LED current at R4 = 50 kOhm; steady on-state current is approximately 5.20 mA.

R3 dissipates approximately 8.92 mW in steady operation, with a maximum saved transient value of 10.19 mW.

Figure 4. R3 power at R4 = 50 kOhm; steady on-state dissipation is approximately 8.92 mW.

# Evidence index

The complete evidence set contains paired PNG and semicolon-delimited CSV exports. Paths below are relative to the repository root.

| ID | Evidence | Files | Repository path (extension omitted) |
| --- | --- | --- | --- |
| 01 | Mid timing, 50 kOhm | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ Timing_range_exports/01_timing_mid_50k |
| 02 | Minimum timing, 1 Ohm | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ Timing_range_exports/02_timing_min_1ohm |
| 03 | Maximum timing, 100 kOhm | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ Timing_range_exports/03_timing_max_100k |
| 04 | LED current, 50 kOhm | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ led_current/04_led_current_mid_50k |
| 05 | Loaded LED voltages | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ loaded_voltage_exports/05_led_loaded_voltages_mid_50k |
| 06 | R3 power | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ r3_power_recording/06_r3_power_mid_50k |
| 07 | Loaded branch composite | PNG + CSV | simulation/kicad/schematic_simulation_response_plots/ loaded_led_exports/07_led_branch_composite |

## Source artifacts

Simulation schematic: simulation/kicad/simulation_schematic/pcb-001-ne555.kicad_sch

Simulator workbook: simulation/kicad/workbook/pcb-001-ne555.wbk

Archived plot package: simulation/kicad/schematic_simulation_response_plots.zip

Timer model: kicad/models/uA555.sub

# Limitations and remaining verification

Reproducibility. Record the exact KiCad application version used.

Acceptance. Define requirement-level criteria before assigning PASS or FAIL status.

Components. Confirm tolerances, temperature sensitivity, LED datasheet limits, and the selected R3 power rating.

Physical gate. Complete fabrication review, assembly inspection, and powered bench measurements before physical-validation closure.

Conclusion: the saved KiCad evidence characterizes stable astable operation across the tested adjustment range and a plausible loaded LED branch. It does not replace requirement-based or physical verification.