PCB-001

Full-Range LTspice Verification Worklog

NE555 Adjustable Astable LED Flasher

Work period: 14 August 2026, 08:29-10:12 (Ireland)

| Stage | Pre-KiCad circuit verification |
| --- | --- |
| Primary objective | Verify the full timing-resistance range and close G1B |
| Sweep values | RB = 10 kOhm, 60 kOhm, 110 kOhm |
| Simulation tool | LTspice 24.1.10 for Windows |
| Final gate result | G1B - Full-Range LTspice Simulation Verification: PASS |

This worklog records the parameter sweep, measurement-method failure, correction, final numerical results, engineering interpretation, and gate decision.

# 1. Session Objective and Starting State

Starting state: the midpoint design had already been verified at RB = 60 kOhm, with LTspice measurements closely matching the analytical NE555 astable equations. The remaining task was to verify the proposed adjustable range at the minimum, midpoint, and maximum effective timing resistance values.

Minimum effective RB: 10 kOhm.

Midpoint effective RB: 60 kOhm.

Maximum effective RB: 110 kOhm.

Goal: confirm timing, frequency, duty cycle, and capacitor switching thresholds at all three points before circuit freeze.

# 2. Parameter Sweep Implementation

R2 was parameterized using {RBval} and the following sweep was applied:

.step param RBval list 10k 60k 110k

The transient simulation remained: .tran 0 5s 0 1ms startup

# 3. Initial Measurement Failure and Root Cause

The first stepped run produced correct period, frequency, and capacitor-threshold values, but invalid negative tHIGH and duty-cycle values for steps 1 and 3. This did not indicate a circuit fault. It exposed a measurement-pairing fault in the original .meas expressions.

| Step | RB | Initial tHIGH | Initial duty | Interpretation |
| --- | --- | --- | --- | --- |
| 1 | 10 kOhm | -0.069535 s | -33.37 % | Invalid - wrong edge pairing |
| 2 | 60 kOhm | 0.485925 s | 53.82 % | Valid |
| 3 | 110 kOhm | -0.764317 s | -47.85 % | Invalid - wrong edge pairing |

Root cause: after TD = 1 s, the first FALL event can occur before the first RISE event. The original direct TRIG/TARG measurement therefore subtracted events in the wrong order for some sweep steps.

# 4. Corrected .meas Method

The measurement strategy was changed from direct edge-pairing to explicit edge timestamps. This made the measurement independent of the oscillator phase at the 1 s analysis cutoff.

Event timestamps

.meas tran TR1 WHEN V(vout)=2.5 TD=1s RISE=1
.meas tran TR2 WHEN V(vout)=2.5 TD=1s RISE=2
.meas tran TF1 WHEN V(vout)=2.5 TD=1s FALL=1
.meas tran TF2 WHEN V(vout)=2.5 TD=1s FALL=2

Derived measurements

.meas tran tHIGH PARAM if(TF1>TR1, TF1-TR1, TF2-TR1)
.meas tran PERIOD PARAM TR2-TR1
.meas tran tLOW PARAM PERIOD-tHIGH
.meas tran FREQ PARAM 1/PERIOD
.meas tran DUTY PARAM 100*tHIGH/PERIOD
.meas tran VCAP_MIN MIN V(vcap) FROM 1s TO 5s
.meas tran VCAP_MAX MAX V(vcap) FROM 1s TO 5s

Key improvement: tHIGH now selects the correct falling edge whether the first post-1 s event is a rise or a fall. tLOW is then obtained from PERIOD - tHIGH, removing unnecessary edge-pair ambiguity.

# 5. Final Full-Range Measurement Results

| RB | tHIGH (s) | tLOW (s) | Period (s) | Freq (Hz) | Duty (%) | VCAP min (V) | VCAP max (V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 kOhm | 0.138819 | 0.069535 | 0.208354 | 4.799530 | 66.6265 | 1.665666 | 3.334334 |
| 60 kOhm | 0.485925 | 0.416961 | 0.902886 | 1.107560 | 53.8191 | 1.665667 | 3.334333 |
| 110 kOhm | 0.833112 | 0.764317 | 1.597429 | 0.626006 | 52.1533 | 1.665668 | 3.334332 |

# 6. Analytical vs LTspice Verification

| RB | Quantity | Analytical | LTspice | Relative error |
| --- | --- | --- | --- | --- |
| 10 kOhm | tHIGH | 0.138600 s | 0.138819 s | +0.158% |
|  | tLOW | 0.069300 s | 0.069535 s | +0.339% |
|  | Frequency | 4.8100 Hz | 4.79953 Hz | -0.218% |
|  | Duty | 66.667% | 66.6265% | -0.060% |
| 60 kOhm | tHIGH | 0.485100 s | 0.485925 s | +0.170% |
|  | tLOW | 0.415800 s | 0.416961 s | +0.279% |
|  | Frequency | 1.1100 Hz | 1.10756 Hz | -0.220% |
|  | Duty | 53.846% | 53.8191% | -0.050% |
| 110 kOhm | tHIGH | 0.831600 s | 0.833112 s | +0.182% |
|  | tLOW | 0.762300 s | 0.764317 s | +0.265% |
|  | Frequency | 0.62739 Hz | 0.626006 Hz | -0.221% |
|  | Duty | 52.174% | 52.1533% | -0.040% |

Verification outcome: all timing and frequency quantities agree with the analytical model to substantially better than 1%. The largest observed relative deviation is approximately 0.34%, occurring in tLOW at RB = 10 kOhm.

# 7. Threshold and Behavioral Interpretation

VCAP minimum remained approximately 1.666 V for all three sweep points.

VCAP maximum remained approximately 3.334 V for all three sweep points.

The timing resistance changes how quickly C1 moves between the thresholds; it does not materially change the thresholds themselves.

The simulated LED flash-rate range is approximately 0.626 Hz to 4.800 Hz.

Duty cycle moves from about 66.6% at the fastest setting toward 50% as RB becomes large relative to RA.

# 8. Gate Decision

| Check | Result | Evidence |
| --- | --- | --- |
| RB sweep executes at 10k / 60k / 110k | PASS | All three operating points converge and oscillate. |
| Frequency agreement | PASS | Approx. 0.22% error at all three points. |
| HIGH-time agreement | PASS | Maximum error approx. 0.18%. |
| LOW-time agreement | PASS | Maximum error approx. 0.34%. |
| Duty-cycle agreement | PASS | Maximum error approx. 0.06%. |
| VCAP thresholds | PASS | Approx. 1/3 VCC and 2/3 VCC across the sweep. |

G1B - Full-Range LTspice Simulation Verification: PASS

Engineering conclusion: the proposed timing network is numerically verified over the intended adjustment range and is suitable to proceed toward final pre-KiCad output-stage verification.

# 9. Engineering Lessons from This Work Block

A failed measurement does not automatically imply a failed design; verification logic must itself be validated.

Stepped simulations can expose phase-dependent weaknesses that are invisible in a single operating-point test.

Explicit event timestamps (TR1/TR2/TF1/TF2) are more robust than assuming a fixed edge order after a measurement delay.

Analytical calculations remain valuable because they provide an independent target against which simulation can be judged.

# 10. Next Work Task

Immediate next task: LED-current verification. Measure the current through D1 / R3 at the operating points and confirm that the 330 Ohm resistor produces an acceptable LED current with the non-ideal NE555 output model.

Measure LED current during VOUT HIGH.

Check peak/steady current against the intended first-pass target.

Confirm R3 = 330 Ohm remains appropriate.

Once output-stage verification passes, freeze the circuit values and begin KiCad schematic capture.