ENGINEERING WORK SESSION RECORD

# PCB-001 NE555 Astable LED Flasher

KiCad schematic capture, simulation verification, PCB import and placement worklog

| Control | Record |
| --- | --- |
| Project | PCB-001 - adjustable NE555 astable LED flasher |
| Session window | 15 August 2026 10:35 BST to 16 August 2026 01:12 BST |
| Active design phase | KiCad 10.0.4 / ngspice; Git repository on branch main |
| Record basis | Referenced task conversation plus repository and file inspection on 16 August 2026 |
| End-of-session gate | Placement review PASS; routing, copper zone and DRC not started |

Outcome  The electrical design, ERC, footprint set and KiCad/ngspice midpoint verification passed. Fourteen physical footprints were imported and iteratively placed into coherent power, timing/core and output zones. The session stopped at placement freeze before any copper was routed.

# Executive status

Table 1 - Session gate summary

| Gate | Scope | Status | Evidence / qualification |
| --- | --- | --- | --- |
| G1D | Prior Tinkercad verification | PASS | Corrected logger re-verified minimum, midpoint and maximum; commit 4ee50f8 was the clean entry baseline. |
| M1 | Native KiCad project scaffold | PASS | KiCad 10.0.4 project created and checkpointed at ed63b92. |
| M2 | Schematic and ERC | PASS | Functional capture completed; final ERC reported 0 errors and 0 warnings. |
| M2B | Physical footprint assignment | PASS | All 14 physical references have through-hole footprints; no blank assignments remained. |
| G1E | KiCad/ngspice schematic verification | PASS | Sustained oscillation confirmed at the 50 kOhm RV1-equivalent midpoint. |
| M3 | PCB import | PASS | Update PCB from Schematic reported 0 warnings and 0 errors and imported 14 physical footprints. |
| M3A | Component placement review | PASS | Placement frozen by visual review; latest reduced outline exists in autosave and is not yet baselined. |
| M3B | Routing and copper | OPEN | 0 segments, 0 vias and 0 zones in the latest inspected board artifact. |
| M4 | DRC / fabrication release | OPEN | No completed DRC, manufacturing package or procurement freeze. |

Handoff condition  The engineering design review reached placement freeze, but the repository is not in a clean handoff state. The tracked production and simulation files currently appear deleted while renamed project files and an autosave are untracked. Reconcile and checkpoint those files before routing.

# 1. Scope, evidence and time basis

This record reconstructs the complete KiCad-focused PCB-001 work session and its enabling repository checkpoints. It covers the clean entry state, project creation, manual schematic capture, ERC resolution, physical package decisions, KiCad/ngspice simulation, separation of production and simulation variants, PCB import, placement iterations and the final handoff state.

Timestamps are British Summer Time (UTC+01:00). The calendar span includes a long pause between the initial project-creation work and the evening capture/layout work; it is not a continuous labour-duration claim.

Table 2 - Evidence basis

| Source | Used for | Reliability / limitation |
| --- | --- | --- |
| Referenced task conversation | Primary chronology, decisions, screen-review results and declared gates. | High for actions recorded in text; attached placement images were not retained in the bounded reference. |
| Git history | Commit identifiers, timestamps, file scope and current branch/remote pointers. | Directly verified in repository. |
| Current KiCad files | Footprint count, routing count, outline geometry, production/simulation content and save-state divergence. | Directly inspected; current worktree is dirty. |
| Stored waveform | KiCad/ngspice transient result at midpoint equivalent. | Directly inspected and embedded as Figure 1. |

# 2. Entry state and session objectives

The repository entered the KiCad phase on branch main with a clean worktree at commit 4ee50f8, test(tinkercad): verify corrected logger across full range. No KiCad project or custom KiCad libraries existed. The authoritative repository already separated docs, simulation and worklogs.

Create native KiCad 10.0.4 project files without fabricating file contents or contaminating earlier verification evidence.

Capture the verified nominal +5 V astable circuit manually, one functional block at a time.

Resolve ERC transparently, then choose real through-hole packages and transfer the physical design to PCB Editor.

Use KiCad/ngspice as an additional schematic-level check while keeping simulation-only helpers out of the manufactured PCB.

Reach a mechanically usable placement freeze before routing.

# 3. Timestamped session chronology

Table 3 - Work-session timeline

| Time (BST) | Stage | Recorded outcome |
| --- | --- | --- |
| 15 Aug 10:35 | Entry review | Clean main branch at 4ee50f8; no KiCad project present; directory and nomenclature plan approved. |
| 15 Aug 11:37 | Project creation | KiCad 10.0.4 scaffold created with 16 symbol instances, no wiring, no footprints and empty PCB. |
| 15 Aug 21:01 | Scaffold checkpoint | Saved native project, staged exactly three KiCad files and prepared controlled commit. |
| 15 Aug 21:06 | Commit ed63b92 | Initial KiCad project scaffold committed. |
| 15 Aug 21:13 | Commit 97b9be5 | Narrow KiCad transient ignore rules committed; design files remained trackable. |
| 15 Aug 21:14-21:58 | Schematic capture | Power, timing, output and test-point networks wired; J1 convention and net labels finalized. |
| 15 Aug 21:58 | ERC gate | First run showed two externally powered-net errors; +5 V and GND PWR_FLAG symbols resolved them; final 0/0 result. |
| 15 Aug 22:00-22:42 | Footprints | All 14 physical references assigned through-hole footprints; post-assignment audit passed. |
| 15 Aug 22:42 | PCB import | Update PCB from Schematic completed with 0 warnings/0 errors; 14-footprint ratsnest imported. |
| 15 Aug 22:55-23:40 | KiCad/ngspice | Simulation readiness issues resolved sequentially; UA555 transient midpoint result passed. |
| 15 Aug 23:51 | Variant separation | Decision made to keep production and simulation schematics separate. |
| 15 Aug 23:55 | Commit e9d7818 | Production KiCad files, simulation variant, UA555 model and waveform evidence checkpointed. |
| 16 Aug 00:03-01:10 | PCB placement | U1 rotated 180 degrees; functional groups tightened; provisional outline reduced; placement review passed. |
| 16 Aug 01:12 | Session close | Routing deferred; next task defined as C3/U1 decoupling followed by VCAP/timing routing. |

# 4. Repository checkpoints and configuration control

Table 4 - Repository checkpoints verified in Git

| Commit | Time (BST) | Subject | Engineering significance |
| --- | --- | --- | --- |
| 4ee50f8 | 15 Aug 10:29 | test(tinkercad): verify corrected logger across full range | Clean entry baseline for the KiCad work. |
| ed63b92 | 15 Aug 21:06 | chore(kicad): initialize PCB-001 project scaffold | Exactly the .kicad_pro, .kicad_sch and .kicad_pcb scaffold files. |
| 97b9be5 | 15 Aug 21:13 | chore(repo): ignore KiCad transient files | Added rules for *-backups, .kicad_prl and ~*.lck while preserving design-file tracking. |
| e9d7818 | 15 Aug 23:55 | Update project files 2026-08-15 23:55:03 | Checkpointed production KiCad changes, UA555 model, waveform evidence and canonical simulation schematic. |

## 4.1 Repository hygiene decision

KiCad backup ZIPs, .kicad_prl preferences and lock files were deliberately treated as transient. The .gitignore change was reviewed and committed separately from the design scaffold. The .kicad_pro, .kicad_sch and .kicad_pcb sources remained explicitly trackable. This created a clean configuration-control boundary before manual wiring.

## 4.2 Current repository state at handoff

Repository risk  At inspection on 16 August, HEAD and origin/main both pointed to e9d7818, but the working tree was not clean. The four tracked production/simulation KiCad paths appeared deleted, while a placement-modified project, its autosave and a simulation schematic under 'New folder' were untracked. The remote alignment indicates that a push occurred after earlier per-checkpoint reports that said no push; that later push was not captured as a separate conversational action.

Tracked paths shown deleted:
  kicad/pcb-001-ne555/pcb-001-ne555.kicad_pro
  kicad/pcb-001-ne555/pcb-001-ne555.kicad_sch
  kicad/pcb-001-ne555/pcb-001-ne555.kicad_pcb
  simulation/kicad/pcb-001-ne555-sim/pcb-001-ne555.kicad_sch

Untracked working artifacts:
  kicad/pcb-001-ne555/pcb-001-ne555-placement-modified.*
  kicad/pcb-001-ne555/_autosave-pcb-001-ne555-placement-modified.kicad_pcb
  simulation/kicad/New folder/pcb-001-ne555.kicad_sch

# 5. KiCad schematic capture

## 5.1 Native project scaffold

The project was created with KiCad 10.0.4 under kicad/pcb-001-ne555/ using basename pcb-001-ne555. The initial schematic contained 16 symbol instances: 14 physical references plus +5 V and GND power symbols. No wires, junctions, net labels, footprints, PWR_FLAG instances or PCB copper existed at the scaffold checkpoint.

## 5.2 Power, reset, control and decoupling block

U1 pin 8 VCC and pin 4 RESET were connected to the global +5 V net.

U1 pin 1 GND was connected to GND.

U1 pin 5 CONT/CV was bypassed to GND by C2 = 10 nF.

C3 = 100 nF was placed across +5 V and GND as the local supply decoupler.

J1 polarity convention was deliberately settled as pad/pin 1 = +5 V and pad/pin 2 = GND.

## 5.3 Astable timing network

+5 V -> R1 10 kOhm -> U1 pin 7 DISCH junction
                    -> R2 10 kOhm -> RV1 100 kOhm rheostat -> VCAP
VCAP -> U1 pin 2 TRIG
VCAP -> U1 pin 6 THRES
VCAP -> C1 10 uF positive; C1 negative -> GND
VCAP -> TP4

RV1 was configured as a rheostat by tying the wiper to one outer terminal. Local VCAP labels replaced long schematic wires, improving auditability without changing connectivity. C1 polarity was reviewed explicitly: positive to VCAP, negative to GND.

## 5.4 Output and test interfaces

The output chain was captured as U1 pin 3 OUT / VOUT to D1, then R3 = 330 Ohm to GND. The final KiCad pad mapping shows D1 pad 2 (anode) on VOUT and pad 1 (cathode) feeding R3 toward GND. An early visual interpretation of LED orientation was rechecked against the actual symbol before the block was accepted.

TP1 = +5 V

TP2 = GND

TP3 = VOUT

TP4 = VCAP

## 5.5 Final pin-by-pin functional audit

Table 5 - NE555 functional pin audit

| Pin | Function | Final net / path | Status |
| --- | --- | --- | --- |
| 1 | GND | GND | PASS |
| 2 | TRIG | VCAP | PASS |
| 3 | OUT | VOUT -> D1 -> R3 -> GND | PASS |
| 4 | RESET | +5 V | PASS |
| 5 | CONT | C2 10 nF -> GND | PASS |
| 6 | THRES | VCAP | PASS |
| 7 | DISCH | R1/R2 junction | PASS |
| 8 | VCC | +5 V | PASS |

# 6. Electrical Rules Check

The first ERC run reported two errors and no warnings. Both errors were 'Input Power pin not driven by any Output Power pins'. This was expected because J1 is a passive connector supplying power from outside the PCB; neither +5 V nor GND contained a power-output pin.

Added one PWR_FLAG to the +5 V net near J1.

Added one PWR_FLAG to the GND net near J1.

Did not suppress or ignore genuine connectivity checks.

Verification result  Final ERC: 0 violations, 0 errors, 0 warnings. M2 / Schematic Complete: PASS.

# 7. Footprint assignment and physical package decisions

The board was intentionally standardized as a through-hole, hand-assembled and probe-friendly first build. All 14 physical references received footprints; the +5 V/GND symbols and PWR_FLAG instances did not transfer as physical parts.

Table 6 - Physical footprint register

| Reference | Assigned footprint | Decision / check |
| --- | --- | --- |
| C1 | Capacitor_THT:CP_Radial_D5.0mm_P2.00mm | 10 uF polarized radial electrolytic; pad 1 positive to VCAP. |
| C2, C3 | Capacitor_THT:C_Disc_D5.0mm_W2.5mm_P5.00mm | 10 nF control bypass and 100 nF decoupler; non-polarized. |
| D1 | LED_THT:LED_D5.0mm | 5 mm through-hole LED; edge visibility prioritized. |
| J1 | TerminalBlock_CUI:TerminalBlock_CUI_TB007-508-02_1x02_P5.08mm_Horizontal | Two-position 5.08 mm screw terminal; wire-entry direction must face edge. |
| R1-R3 | Resistor_THT:R_Axial_DIN0207_L6.3mm_D2.5mm_P10.16mm_Horizontal | Standard 0.25 W axial through-hole package. |
| RV1 | Potentiometer_THT:Potentiometer_Bourns_3386P_Vertical | 100 kOhm single-turn trimmer; screwdriver clearance required. |
| TP1-TP4 | TestPoint:TestPoint_THTPad_D2.0mm_Drill1.0mm | Plated through-hole probe pads; not assembled BOM items. |
| U1 | Package_DIP:DIP-8_W7.62mm | Through-hole 8-pin timer package; intended purchase discussed as NE555P-class PDIP. |

Procurement caveat  Footprint assignment is a geometry decision, not a final part-number freeze. Actual capacitor diameter/pitch, terminal-block orientation, trimmer pinout, LED lead polarity and NE555 package must be verified against selected manufacturer datasheets before fabrication.

# 8. PCB import

After saving the footprint assignments and rechecking the schematic, Update PCB from Schematic was run. The update dialog reported 0 warnings and 0 errors, with relinking, symbol-defined footprint replacement and field updates enabled. Destructive options such as deleting footprints with no symbols remained disabled.

Fourteen physical footprints were imported as an initial overlapping cluster and dropped into clear working space.

Cyan ratsnest lines were treated as connectivity feedback, not routed copper.

The design explicitly checked that V1 and R4 simulation helpers were not proposed for the PCB.

A provisional 50 mm x 50 mm Edge.Cuts rectangle was used as working space, not as a final size commitment.

# 9. KiCad/ngspice verification

## 9.1 Simulation-readiness issues resolved

Table 7 - Simulation setup issue log

| Item | Observed condition | Resolution |
| --- | --- | --- |
| Physical-only connector/test points | J1 and TP1-TP4 had no SPICE models. | Excluded from simulation. |
| Power source | +5 V symbols and PWR_FLAG do not generate voltage. | Added simulation-only V1 = 5 V DC source. |
| Passive value syntax | Human-readable values such as '10 kOhm' caused ngspice parsing errors. | Used SPICE notation: 10k, 330, 10u, 10n, 100n in the simulation variant. |
| LED model | The 'red LED' value did not resolve to a valid SPICE model. | Excluded D1 and R3 for the oscillator-only KiCad check; loading had already been verified elsewhere. |
| Potentiometer | Installed KiCad UI did not expose the expected potentiometer subtype path. | Excluded RV1 from SPICE and added R4 = 50k as the midpoint equivalent; R4 excluded from board/BOM/position files. |
| NE555 model | Timer:NE555D had no default subcircuit. | Attached self-contained uA555.sub, selected model UA555 and verified 1:1 pins 1-8. |

## 9.2 Transient analysis setup

Analysis: TRAN (transient)
Time step: 1 ms
Final time: 10 s
Initial time: 0 s
Maximum time step: 1 ms
Use initial conditions: unchecked
Saved data: all voltages; currents/power/digital events not required
Plotted signals: V(VCAP), V(VOUT)
Compatibility: PSpice and LTspice

## 9.3 Verification result and interpretation

Table 8 - KiCad/ngspice midpoint result

| Measure | Observed / configured value |
| --- | --- |
| Supply | 5 V |
| RA | 10 kOhm |
| Effective RB | R2 10 kOhm + R4 50 kOhm = 60 kOhm |
| Timing capacitance | 10 uF |
| VOUT | Approximately 0 to 4.8-4.9 V square wave |
| VCAP | Approximately 1.8 to 3.3 V cyclic charge/discharge |
| Period | Approximately 0.80 s |
| Frequency | Approximately 1.24-1.25 Hz |
| Duty cycle | Approximately 55% |
| Gate | G1E PASS - sustained astable oscillation confirmed |

The ideal textbook estimate for RA = 10 kOhm, RB = 60 kOhm and C = 10 uF was about 0.901 s / 1.11 Hz, assuming exact one-third and two-thirds supply thresholds. The transistor-level UA555 model switched nearer 1.8 V and 3.3 V. Recalculating with those observed thresholds yields approximately 0.807 s / 1.24 Hz, closely matching the trace. The difference was therefore recorded as model behaviour, not a failed circuit.

Figure 1 - Stored KiCad/ngspice midpoint transient evidence (blue VOUT, red VCAP).

Evidence file: kicad/evidence/midpoint-transient-2026-08-15.png

# 10. Production / simulation variant separation

After the successful transient run, the production design and KiCad simulation schematic were deliberately separated. The production schematic remains the manufacturing source; the simulation variant retains the analysis helpers and model assignments.

Table 9 - Variant responsibilities

| Variant | Required contents | Configuration rule |
| --- | --- | --- |
| Production schematic | J1 external 5 V input; physical RV1 100 kOhm; D1/R3 output; all four test points; 14 board footprints. | No V1; no R4; no simulation helpers in PCB/BOM. |
| Simulation schematic | V1 = 5 V; R4 = 50k midpoint substitute; UA555 model; physical-only exclusions as needed. | R4 excluded from board/BOM/position; V1 simulation-only; preserve waveform and model. |

Portability risk  The currently inspected simulation schematic stores an absolute Windows path to uA555.sub. Convert that assignment to a stable project-relative model path during repository cleanup, then rerun the transient check.

# 11. PCB placement iterations and layout decisions

Table 10 - Placement iteration log

| Iteration | Change | Decision rationale |
| --- | --- | --- |
| A - Import / provisional | 14-footprint cluster and 50 x 50 mm working outline. | Establish functional zones; do not route. |
| B - Functional zoning | J1/TP1/TP2 power area; R1/R2/RV1/C1/TP4 timing area; U1/C2/C3 core; R3/D1/TP3 output area. | Shorten local nets without chasing visual symmetry. |
| C - U1 orientation | U1 rotated 180 degrees. | Place pins 6/7 toward timing, pin 3 toward output, and supply pins where C3 can sit tightly. |
| D - Decoupling/control | C3 moved immediately below/next to U1 supply pads; C2 moved close to pin 5. | Electrical loop length took priority over aesthetic alignment. |
| E - Timing consolidation | R1/R2 pulled toward RV1/U1; C1 and TP4 tightened around VCAP. | Keep VCAP short and local; preserve RV1 screw access. |
| F - Mechanical edges | J1 retained at an accessible edge; D1 exposed at the edge; TP3 near output. | Verify terminal entry in 3D and preserve LED visibility/probe access. |
| G - Outline reduction | Placement compacted and Edge.Cuts pulled inward. | Stop shrinking before routing consumes clearance; leave roughly 2-3 mm ordinary edge clearance. |
| H - Placement freeze | Power across top, timing left, core center, output right; C3 below U1 and TP4 adjacent to timing node. | M3A PASS by review; no further broad shuffling before routing. |

## 11.1 Final placement architecture

POWER / ENTRY          TIMING + CORE                 OUTPUT
J1, TP1, TP2           R1, R2, RV1, C1, TP4          R3, D1, TP3
                       U1, C2, C3

Layout priorities:
  C3 supply loop: extremely short
  VCAP: short and separated from VOUT
  J1: cable entry facing board edge
  RV1: unobstructed screwdriver access
  D1: visible at board edge
  Test points: reachable with practical ground-lead geometry

## 11.2 Outline and save-state evidence

Table 11 - Board outline evidence

| Artifact | Edge.Cuts rectangle | Timestamp | Interpretation |
| --- | --- | --- | --- |
| Named placement-modified board | 50.00 x 50.00 mm | Saved 16 Aug 01:00 | Older/provisional outline. |
| Newest autosave | 39.25 x 41.75 mm | Saved 16 Aug 01:10 | Latest reduced outline and likely final placement state. |

Do not route yet  Open and review the newest autosave, then deliberately save the approved placement into the authoritative named board file and checkpoint it. Routing against the older 50 x 50 mm saved file would lose the final outline decision.

# 12. Current board status

Table 12 - End-of-session board state

| Item | Observed state | Status | Qualification |
| --- | --- | --- | --- |
| Physical footprints | 14 | PASS | All expected production references present. |
| Board outline | 39.25 x 41.75 mm in latest autosave | HOLD | Reduced outline is not yet in the named saved board. |
| Routed segments | 0 | OPEN | Routing intentionally not started. |
| Vias | 0 | OPEN | None required/placed yet. |
| Copper zones | 0 | OPEN | GND zone planned after explicit routing. |
| DRC | Not run to release criteria | OPEN | Expected because the board is unrouted. |
| Mounting holes | Decision pending | OPEN | Must be resolved before traces occupy remaining space. |
| Fabrication outputs | None | OPEN | No Gerbers, drill set, BOM or position files released. |

# 13. Open risks and required controls

Table 13 - Risk register

| ID | Level | Risk | Control / next action |
| --- | --- | --- | --- |
| R1 | High | Repository paths diverge from HEAD: authoritative tracked KiCad files appear deleted and renamed/untracked copies exist. | Reconcile paths, inspect diffs, stage intentionally and create a clean checkpoint before design edits. |
| R2 | High | Latest placement and 39.25 x 41.75 mm outline appear only in an autosave. | Open autosave, visually compare, save approved state under authoritative basename, then verify counts and outline. |
| R3 | High | Simulation variant is currently under an untracked 'New folder' path while the canonical tracked path appears deleted. | Move/Save As to simulation/kicad/pcb-001-ne555-sim/ and verify production/simulation separation. |
| R4 | Medium | UA555 assignment uses an absolute machine-specific model path. | Use a project-relative model reference and rerun ngspice after relocation. |
| R5 | Medium | No routing, ground zone or DRC has been completed. | Route in controlled priority order, add GND zone, then resolve every DRC finding. |
| R6 | Medium | Mechanical suitability is not frozen to exact purchased part numbers. | Check datasheets for C1/C2/C3 pitch/body, RV1 pinout, J1 orientation, LED polarity and DIP part. |
| R7 | Medium | Mounting-hole requirement is unresolved. | Decide before routing; if required, place holes and keepouts now. |
| R8 | Low | KiCad oscillator simulation excluded LED/R3 loading. | Treat G1E as oscillator verification; retain LTspice/Tinkercad evidence for loaded output behaviour. |
| R9 | Low | Attached placement screenshots are not preserved as repository evidence. | Capture and version a final 2D PCB Editor image and, optionally, a 3D view after placement save. |

# 14. Next-session execution plan

Table 14 - Ordered next-session actions

| Step | Action | Execution detail | Exit criterion |
| --- | --- | --- | --- |
| 1 | Preserve the latest placement | Open the 01:10 autosave and the 01:00 named board side by side; confirm the intended placement and 39.25 x 41.75 mm outline. | Approved placement is safely saved under the authoritative project basename. |
| 2 | Reconcile Git paths | Restore or intentionally rename the production project and canonical simulation variant; remove the accidental 'New folder' naming through normal save/move workflow. | git status shows only intended changes; no authoritative source is shown deleted unintentionally. |
| 3 | Revalidate production schematic | Confirm no V1 or R4 in production; verify 14 physical footprints, J1 polarity, C1/LED polarity and all net labels; run ERC. | ERC = 0 errors, 0 warnings; PCB update proposes no simulation helpers. |
| 4 | Revalidate simulation variant | Make uA555.sub path portable; confirm V1/R4 and all exclusions; rerun 1 ms / 10 s transient. | VOUT/VCAP oscillation reproduces the recorded midpoint result. |
| 5 | Close mechanical decisions | Decide mounting holes; inspect F.Courtyard overlaps, J1 cable-entry direction, RV1 adjustment clearance and D1 visibility in 2D/3D. | Mechanical review recorded PASS before copper. |
| 6 | Route C3 decoupling first | Use short/wide geometry between U1 supply pads and C3. Do not allow cosmetic placement to lengthen the loop. | C3/U1 loop is minimal and reviewable. |
| 7 | Route VCAP/timing | Route pins 2/6, C1, RV1 and TP4, then the pin 7 / R1 / R2 timing path. Keep VOUT away from VCAP. | Timing nets are short, local and not parallel to the switching output for meaningful distance. |
| 8 | Route output and power | Route VOUT -> R3 -> D1 / TP3, then +5 V distribution and remaining signals. | Power and output paths match the intended functional zoning. |
| 9 | Add GND copper zone | Use an appropriate ground zone after deliberate routing; refill and inspect thermal reliefs and isolated islands. | Ground connectivity is complete and zone behaviour is visually reviewed. |
| 10 | Run DRC and release checks | Resolve all unrouted/clearance/edge/silkscreen issues; inspect polarity and connector markings; generate fabrication data only after sign-off. | DRC clean; manufacturing review and exact part-number checks complete. |

## 14.1 Routing design rules agreed in-session

Table 15 - Initial routing constraints

| Net/class | Initial rule | Rationale |
| --- | --- | --- |
| Ordinary signals | 0.25-0.30 mm | Use for low-current signals unless later DRC/fabrication constraints require adjustment. |
| +5 V | 0.50 mm | Wider than ordinary signals. |
| Explicit GND traces | 0.50 mm | Use where routed; a GND copper zone will be added later. |
| VCAP | Topology rule | Keep short; avoid meaningful parallel runs beside VOUT. |
| C3 decoupling | Priority rule | First route; shortest practical local supply loop. |

# 15. Session close and sign-off statement

The session achieved the intended design-development objective: a cleanly audited NE555 astable schematic, ERC pass, complete through-hole footprint set, independent KiCad/ngspice midpoint verification, physical PCB import and a coherent placement architecture ready for routing. The electrical and placement decisions are sufficiently mature to continue, but the latest placement must first be recovered from autosave and the production/simulation file paths must be reconciled into a clean repository checkpoint.

Formal end state  M3A Component Placement: PASS by engineering review. Configuration baseline: HOLD pending save/repository reconciliation. M3B Routing, GND zone, DRC and fabrication release: NOT STARTED.

# Appendix A - Evidence register

Table 16 - Evidence register

| Type | Identifier / path | Purpose |
| --- | --- | --- |
| Git | 4ee50f8 | Clean verified Tinkercad entry baseline. |
| Git | ed63b92 | Initial native KiCad scaffold. |
| Git | 97b9be5 | KiCad transient-file ignore policy. |
| Git | e9d7818 | Production schematic/PCB update, UA555 model, waveform and simulation schematic checkpoint. |
| Model | kicad/models/uA555.sub | Self-contained UA555 subcircuit; declaration .SUBCKT UA555 1 2 3 4 5 6 7 8. |
| Waveform | kicad/evidence/midpoint-transient-2026-08-15.png | Stored transient evidence embedded as Figure 1. |
| Production working files | kicad/pcb-001-ne555/pcb-001-ne555-placement-modified.* | Untracked renamed project at handoff; requires reconciliation. |
| Latest board autosave | kicad/pcb-001-ne555/_autosave-pcb-001-ne555-placement-modified.kicad_pcb | 14 footprints, 39.25 x 41.75 mm outline, 0 segments/vias/zones. |
| Current simulation working file | simulation/kicad/New folder/pcb-001-ne555.kicad_sch | Untracked simulation variant with V1/R4 and UA555 assignment. |

# Appendix B - Status legend

| Status | Meaning |
| --- | --- |
| PASS | Completed and supported by recorded evidence. |
| HOLD | Engineering content is acceptable, but configuration or save-state work is required before continuation. |
| OPEN | Required work has not yet been completed. |